PAGE  
xi

Design of an Analog Memory Cell 

in 0.25 micron CMOS process

Thesis submitted in partial fulfillment 

of the requirements for the degree of 

Bachelor of Technology (Honours) 

in 

Electronics and Electrical Communication Engineering

by

Paramita Barai
under the guidance of

Dr. A. S. Dhar

[image: image1.png]



Department of Electronics and Electrical Communication Engineering

Indian Institute of Technology

KHARAGPUR

2002
Certificate

This is to certify that this thesis titled “Design of an Analog Memory Cell in 0.25 micron CMOS process” submitted by Paramita Barai to the Department of Electronics and Electrical Communication Engineering, Indian Institute of Technology, Kharagpur, in partial fulfillment of the requirements for the award of Bachelor of Technology (Honors) in 2002 is an authentic record of the work carried out by Paramita Barai under my guidance and supervision.

In my opinion, this work fulfills the requirements for which it has been submitted. This thesis has not been submitted to any other university / institution for any degree / diploma.

Dr A. S. Dhar

Abstract

CMOS VLSI technology is the most dominant integration methodology prevailing in the world today. Various signal-processing blocks are made using analog or digital design techniques in MOS VLSI. 


An important component is the Memory unit used to store data. In the project a memory cell has been built up using analog design method. A capacitor is used as the basic storage device. The main idea behind analog memory is that the analog value of the charge or voltage stored in the capacitor is the data stored. So the dielectric quality of the capacitor becomes important here to determine how effectively it can store some charge. Analog memory is a trade off between hardware cost, chip area and accuracy or quality of storage. 


The circuit of analog memory cell was developed starting from the idea that required voltage will be stored in a capacitor and MOS transistors were used as switches. A given technology of integration was used and hence the dielectric property of the capacitor was fixed. By suitable circuit configuration the analog voltage value was written to the capacitor, read out when required and the charge loss was also refreshed. 

The results obtained are as given in the thesis.    

Contents

List of figures ………………………………………………………………………..vii

List of tables …………………………………………………………………………..x

1. Introduction








                1

     1.1 Analog Integrated Circuit Design ……………………………………………..2

1.2  CMOS Technology …………………………………………………………...4

1.3  Comparison of CMOS and Bipolar analog IC ………………………………..6

1.4  Analog Memory using CMOS Technology …………………………………..6

1.5  Organization of the Thesis ……………………………………………………7

2. Background









    8

2.1 Reason for choosing Analog Design ……………………………………………8

2.2 Analog Sampled System ………………………………………………………10

2.3 Review of some previous works ………………………………………………15

2.3.1 Analog Memory Elements ………………………………………………...15

2.3.2 Simoni et al.'s Optical Sensor and Analog Memory Chip with Change Detection ………………………………………………………………...17
3. Design of the basic Memory Cell : with writing and reading facility only 
  19

    System Specifications …………………………………………………………….19

    3.1 Initial Idea to store charge in a capacitor ……………………………………..20

          3.1.1 Circuit …………………………………………………………………..20  

          3.1.2 Simulation and Results ………………………………………………....21

    3.2 Introducing another capacitor for reading out stored voltage ………………...24

          3.2.1 Circuit …………………………………………………………………..24

          3.2.2 Simulation and Results …………………………………………………25

    3.3 Discharging the secondary capacitor to prevent it’s leakage charging ……….29

           3.3.1 Circuit ………………………………………………………………….29

           3.3.2 Simulation and Results ………………………………………………...31

4. Design of self-refreshing Analog Memory Cell




  35

    4.1 Reading voltage stored from secondary capacitor …………………………....35

          4.1.1 Circuit …………………………………………………………………..35

          4.1.2 Simulation and Results ………………………………………………....37

    4.2 Final Analog Memory Cell with refreshing arrangement …………………….41

          4.2.1 Circuit …………………………………………………………………..41

          4.2.2 Simulation and Results ………………………………………………....43

     Results ……………………………………………………………………………50

    4.3 Layout









  53

           4.3.1 Circuit Schematic for Layout..………………………………………….53

           4.3.2 Layout (Partial) of the analog memory cell …………………………....54

5.Conclusions ………………………………………………………………………..55

Appendix 

A. CMOS Device Modeling for Circuit Design ………………………………..57

A.1 Simple MOS large signal model ………………………………………..57

A.2 Simple MOS Small – Signal Model …………………………………….61

B. The Cadence IC 442 set of tools …………………………………………….62

B.1 Design Entry …………………………………………………………….62

B.2 Simulation Environment ………………………………………………..62

B.3 Design Synthesis and Layout …………………………………………...63

B.4 Design Verification ……………………………………………………..63

C. Acronyms ……………………………………………………………………64

Bibliography ………………………………………………………………………...65

List of Figures

1.1 Flowchart of analog integrated circuit development ………………………..3

1.2 Categories of Silicon Technology …………………………………………...5

2.1   Typical Digital Processing Block …………………………………………...8

2.2   Analog Signal Processing Block …………………………………………...10

2.3   A 256 pixel * 256 pixel Image ……………………………………………..11

2.4   DRAM structure for storing 8 bit digital word ……………………….……11

2.5   Analog memory structure ………………………………………………….12

2.6   A DRAM style memory for storing analog charge ………………………...15
2.7   Circuit for reducing the leakage ……………………………………………16
2.8   Schematic of a cell of Simoni et al.'s motion detection chip ………………18
3.1         Main scheme of storing charge in a capacitor in analog domain …………..20

3.2         Signal Value or Input voltage ( Vin ) to be stored ………………………….22

3.3         Write Control Signal (Vwrite ) ………………………………………….…...22

3.4         Voltage across Cprimary………………………………………………….…...23

3.5         Circuit with secondary capacitor for memory read out ……………………24

3.6         Voltages across primary and secondary capacitors ………………………..26

3.7         Vtransfer, Signal controlling transfer of voltage between primary and secondary Capacitors ……………………………………………………………27

3.8         Charge sharing between primary and secondary capacitors …………….…27

3.9         Voltage fall across primary capacitor ……………………………………...28

3.10   Voltage across secondary capacitor ………………………………………..28

3.11   Circuit with discharging of secondary capacitor …………………………..30

3.12       Signal Vtransfer applied ……………………………………………………...31
3.13       Signal Vdischarge applied to discharge Csecondary when not required ………....32
3.14       Voltage across Csecondary …………………………………………………….32

3.15       Voltage fall in Cprimary  ……………………………………………………...33

3.16       Comparison of voltages across primary and secondary capacitors ………..34

4.1 Analog memory cell with reading from and writing to memory ………….36

4.2         Input and Output voltages of op amp ………………………………………39

4.3         Voltages across primary and secondary capacitors as compared to the voltage    

               amplified by the op amp …………………………………………………..40

4.4 Final Circuit of Analog Memory Cell ……………………………………..42

4.5         Vin, Input pulse given……………………………………………………….43

4.6         Vwrite, Pulse controlling writing of input voltage to primary capacitor …….44

4.7         Voltage across the primary capacitor ……………………………………....44

4.8
Control signal Vtransfer given to transfer charge from primary to secondary capacitor ……………………………………………………………………..45

4.9         Voltage across the secondary capacitor …………………………………....45

4.10 Voltage Vdischarge given to discharge secondary capacitor when charge is not 

              stored in it ………………………………………………………………….46

4.11      Amplified voltage from op amp ie. the voltage read out …………………..46

4.12      Control voltage Vrefresh ……………………………………………………...47

4.13      Various voltages when Vin = 0.4 volts ……………………………………..48

4.14      Various voltages when Vin = 1 volts ……………………………………….49

4.15      Various voltages when Vin = 1.9 volts ……………………………………..50

List of Tables

4.1 Determining value of R1 from Gain required …………………………………...38

4.2 Quality of storage of analog voltage in the memory ( as % error ) ……………..51

A.1 Constants for Silicon ……………………………………………………………59

Acknowledgements

I am grateful to my supervisor Prof A. S. Dhar, for his encouragement and support throughout the project. I am indebted to him for providing helpful hints and suggestions at every stage of my work. I have gained immeasurably from his vast knowledge and experience. 

I would like to thank my classmate Rajarshi Mukherjee in helping me regarding working with the software. He has helped me with learning the software tools that are used for design of integrated circuits and also have provided unhindered assistance in the understanding of the design process.

I would like to thank specially the IIT Foundation for setting up the Advanced

VLSI Design Laboratory, which made my project possible. Special thanks to Dr. Bijoy G. Chatterjee of National Semiconductor for his help to the laboratory right from conception of the idea to the present state. I would also like to thank the employees at National Semiconductor's India Design Centre, Bangalore for their kind help in the setting up of the software environment. 

I would also like to thank Samiran – da and Uttam – da for their service and help in working in the VLSI laboratory.

Paramita Barai

